考虑飞行能耗的无人机三维路径规划：基于改进的椭圆切线图的方法

摘 要：针对无人机路径规划中能量消耗优化不足的问题，提出一种基于切线图并考虑能耗的无人机三维路径规划方法。首先，采用椭圆柱体模型对不规则障碍物进行安全包络建模，在满足安全距离的同时降低计算冗余；其次，通过双向搜索策略和节点筛选机制对椭圆切线图法进行改进，以生成二维基准路径点；最后，耦合二维基准路径点和能量消耗模型生成三维场景下的无人机路径规划方法。构建了三种不同障碍物密度和布局的测试场景，分别与A*算法、RRT算法、PSO算法和VFH算法进行对比分析。实验表明，相较于A*、RRT、PSO和VFH等算法，其平均路径长度分别减少了8.7 %、18.7 %、13.4 %和4.1 %，转向次数降低了68.8-82.8 %，能量消耗缩减了34.0-59.1 %，有效解决了传统方法在路径长度、运动平滑性与能量消耗间的权衡难题，为提升无人机任务续航能力提供了兼具理论创新与工程实用性的解决方案。

关键词：无人机；三维路径规划；切线图；能量消耗；避障

中图分类号：X915.5  文献标志码：A

近年来，随着自主控制技术的突破性进展，无人驾驶飞行器（unmanned aerial vehicle, UAV）在交通检查、灾难救援、货物运输、和目标侦察等领域得到了广泛应用[1]。路径规划作为无人机自主控制的关键组成部分，对实现无人机的有效应用至关重要。然而，在障碍物密集、不确定且复杂的环境中，如何在满足避障要求、轨迹可行性、实时规划能力及满意路径长度等前提下实现高效路径规划仍然是一个巨大的挑战。到目前为止，已有四种路径规划方法，即基于图的方法、基于抽样的方法、势场方法和智能优化方法[2]。

基于图的方法包括Voronoi图、A*算法和切线图等。Voronoi图通过自由空间划分提升规划效率，但其路径偏离最优。学者们通过结合马尔可夫决策过程[3]、Dijkstra算法[4]、加权的Voronoi图和粒子群算法[5]等方法改进路径长度和安全性。A*算法凭借启发式搜索在复杂环境中表现优异，针对节点扩展和存储问题，部分研究提出了节点改造[6]、双向搜索与势场算法结合[7]等优化策略以提升算法速度。切线图法[8]通过寻找凸多边形间的公共切线来规划路径，但路径曲折且接近障碍物。已有研究引入圆形[9]和椭圆形包络[10]以优化路径平滑性，椭圆模型在狭窄空间更具优势，但其应用局限于二维场景。

基于抽样的方法主要包括概率路线图法（probabilistic roadmap method, PRM）和快速探测随机树（rapidly-exploring random trees, RRT）法。PRM算法通过随机采样构建路线图，但其两点边值问题导致计算成本较高。Li等[11]通过采样策略优化与路径平滑以提升算法效率。RRT算法凭借随机树扩展实现快速搜索，但其本质上的随机性限制了其在环境中的性能。姚信宇等[12]在节点扩展过程中引入引力-斥力场约束增强了避障能力和导向性。

势场算法有人工势能场方法（artificial potential field, APF）和虚拟力场法（virtual force field, VFH）。Hao等[13]为优化路径经济性与避障效能，在APF中融合碰撞风险评估与目标导向机制。Borenstein[14]针对APF的局部振荡问题，提出基于极性直方图的VFH算法，通过实时出口探测增强避障鲁棒性。Ding等[15]进一步融合环境感知与运动学约束优化VFH，显著提升路径搜索效率并降低了误检概率。

智能优化算法包括遗传算法（genetic algorithm, GA）、蚁群算法（ant colony optimization, ACO）和粒子群优化（particle swarm optimization, PSO）等。具有全局优化能力，但在复杂环境中需要耗费大量时间。为此，Li[16]等提出了自适应遗传算法适用于复杂战场中隐身无人机的路径规划。Liu等[17]通过改进蚁群算法避免算法陷入局部最优。Deng等[18]结合粒子群与遗传算法优势，通过动态惯性权重和S形函数优化，提升了无人机三维路径规划的效率。

尽管上述方法在避障能力、路径长度和计算效率等方面取得进展，但普遍忽视能量消耗这一关键问题。无人机续航受电池容量限制，能量效率直接影响任务可靠性，以最低能量消耗为目标进行路径规划已成为重要挑战[19]。为此，本文提出一种基于切线图并考虑能耗的无人机三维路径规划方法。首先，为节省计算资源将障碍物建模为椭圆柱体；其次，基于改进的椭圆切线图法生成二维基准路径点；最后，结合二维基准路径点和能耗模型，提出三维路径规划方法。实验表明，该方法相较于其他四种路径规划方法，在路径长度、转向次数和能量消耗等方面展现出显著优势，证明了其有效性和实用性。

1 环境建模

假设城市各建筑物的位置、高度等参数均已知。考虑到现代建筑物多为不规则的多边形形状，会导致计算量大、路径不光滑和近障碍物等问题。为了克服上述缺点，从俯视角度出发将建筑物统一建模为椭圆形，可以描述各种障碍物，便于后续的路径规划操作。基于上述考虑，无人机路径规划问题可以定义为：在给定的飞行条件和飞行环境中，为无人机规划一条从起点S出发，避开环境中N个障碍物到达终点T的可行的、无碰撞的和最优的路径。建筑物建模如图1所示。

图1 建筑物建模示意图

障碍物的数学表示如下式所示：

（1）

上述公式设定了无人机飞行的禁区。其中和分别表示椭圆的半长轴和半短轴；表示半长轴的倾角；是无人机与障碍物之间所需的最小安全距离，它保证了路径的安全性；(,)表示障碍物k的中心坐标；表示障碍物k的高度。

基于上述公式，无碰撞路径上的每个路径点都应满足以下公式：

（2）

除上述限制外，无人机路径规划必须满足以下几点约束：①最大范围约束[19]。在整个飞行过程中，无人机的航程受燃料量的限制。每个路径点航程的总和应小于最大飞行范围。②最小转弯半径[20]。无人机的航向可以通过调整方向舵和副翼的角度来改变。然而，由于惯性效应，航向的改变需要时间和转弯半径，因此在路径规划中需要考虑最小转弯半径。③通信约束[21]。对于与通讯设备相连的无人机，其路径应确保稳定的通信和限制干扰。

2 二维场景下的椭圆切线图法

2.1 椭圆切线图法

椭圆切线图法[10]是一种基于图的改进方法，将障碍物用椭圆形替代，该方法消除了大量图构造需求。通常，无碰撞路径可以通过两个步骤生成：路径点生成和更新起点和终点。椭圆切线图法原理如图2所示，点S是起点，点T是终点。直线路径ST首先与障碍物1碰撞。可以从S点生成对障碍物1的两条切线；类似地，从T点生成两条切线，四条切线相交于N1和N1′两点。因此，产生了两个临时路径(即S→N1→T和S→N1′→T)。子路径N1′T撞击障碍物2，从点N1′和点T出发，对障碍物2生成四条切线，四条切线的交点记为N2和N2′；同理，N1T首先撞击障碍物3，从点N1和点T出发，对障碍物3生成四条切线，四条切线的交点记为N3和N3′。通过从两条临时路径中选择一条子路径来逐步实现路径生成，该方法遵循以下优先规则：①选择起点切线碰撞障碍物较少的子路径。②选择终点切线碰撞障碍物较少的子路径。③选择长度较短的子路径。此时，两个临时路径分别为S→N1′→N2′→T和S→N1→N3′→T，直线路径N2′T与障碍物5碰撞，N3′T与障碍物4发生碰撞；重复上述操作直至到达终点T，可生成从起点S到终点T的无碰撞路径。路径分别为S→N1′→N2′→N5′→T和S→N1→N3′→N4→T；根据规则选择较短的S→N1→N3′→N4→T为无人机的飞行路径。

图2 椭圆切线图法原理

2.2 改进的椭圆切线图法

椭圆切线图法的优势在于能够将不规则的障碍物包络为椭圆，从而减少路径规划的冗余度，节省计算资源。然而，在实际应用中，如从S→N1→N3′→N4→T的路径所示，该方法生成的路径并非总是最优的。相比之下，S→N3′→N4→T的路径在路径长度和转向次数上更优。频繁的转向不仅增加路径长度，还影响了路径的平滑性，这对无人机路径规划至关重要。基于上述问题，笔者提出了一种改进的椭圆切线图法以克服原方法在路径长度和频繁转向上的不足。椭圆切线图法采用从起点S出发，避开中间障碍物到达终点T的单向路径规划搜索策略。而改进的椭圆切线图法引入双向搜索策略来进行路径规划：同时从起点S和终点T开始搜索，两者互为目标点。按照切线相交的原则，正向和反向同时进行搜索，找到从S到T及从T到S的最佳路径。在此过程中，算法程序会标记所有无碰撞路径点，筛选路径节点并输出一条既考虑转向次数又优化路径长度的综合最优路径。算法1提供了改进的椭圆切线图法的伪代码实现，主要程序如表1所示。

对改进的椭圆切线图法进行复杂度分析，算法中的计算复杂度主要用来解决避障问题。假设障碍物的数量为N，且避开单个障碍物的计算复杂度为O（n），其中n代表与单个障碍物相关的计算成本因素（例如障碍物形状、大小及相对于路径的位置等）。由于改进的椭圆切线图法采用了双向搜索策略，在最坏的情况下，算法需要从两个方向分别处理每个障碍物以确保找到最优路径。因此，整个算法的计算复杂度在最坏情况下为O（2N×n）。

表1 改进的椭圆切线图法的伪代码

算法1：改进的椭圆切线图法

Input: 起点S，终点T，障碍物集合N Output: 最短无碰撞路径ShortestPath 1.根据公式（1）生成每个障碍物的最小外接椭圆 2. Initialize: Path ←，C←，B←∅ 3. while C ≠ ∅ do 4. O←Path(end)，D←C(end) 5. 连接OD，判断OD是否与障碍物发生碰撞 6. if OD无碰撞 then 7. 添加D to Path，删除D from C 8. else 9. 生成O和D到首个碰撞障碍物的切线交点N1和N1′ 10. 根据规则选择更优的子路径N，判断ON是否与障碍物发生碰撞 11. if ON 无碰撞 then 12. Path←Path∪ 13. else 14. C←C∪，B←B∪{碰撞障碍物} 15. end if 16. end if 17. end while 18. return Path 19. 从S到T正向搜索 20. 从T到S反向搜索 21. 合并正反向搜索中的所有无碰撞路径点 22. return 按路径长度排序后的最短无碰撞路径ShortestPath

3 无人机三维路径规划方法

切线图法作为经典路径规划方法，在二维空间中通过构造障碍物切线来实现避障，在无人机最短无碰撞路径求解中具有显著优势。本文第2节提出的改进椭圆切线图法在二维场景下展现出了更优的性能，具体算法对比验证详见第4节。然而，在三维空间路径规划中，系统需同时满足以下约束条件：①水平面的障碍规避；②垂直方向高度层障碍物分布；③地形高程动态变化。此外，考虑到无人机有限的燃料携带量，路径规划过程还需将能量消耗作为关键优化目标。

3.1 基于切线图并考虑能耗的三维路径规划方法

针对上述多维约束问题，提出一种基于切线图并考虑能耗的三维路径规划（energy consumption aware 3d path planning based on tangent graph, 简称ECTG）方法。当起点和终点处于同一高度时，ECTG采用降维策略，将三维规划转化为二维平面问题，利用改进的椭圆切线图法进行求解。对于存在高度差的起点和终点，基于改进的椭圆切线图法生成的二维基准路径点为基础，构建三维参考路径点。当检测到路径段存在障碍物碰撞风险时，建立能量消耗评估模型：比较水平绕飞与爬升穿越两种策略的能量消耗。若路径点有n个，则最多可以有个路径备选，计算所有备选路径的能量消耗，输出能量消耗最低的路径方案。该复合策略不仅保留了切线图法的高效性，还通过在三维空间中优化能量消耗显著提升了路径规划的质量。无人机三维路径规划方法的具体算法流程图如图3所示。展示了从路径生成到考虑能量消耗全过程的系统处理。

图3 ECTG流程图

3.2 无人机能耗模型

无人机运行时的电池输出功率与空气密度相关。在距海平面以下高度H小于11000m的高度下，温度和空气密度随高度的增加而减小，其变化规律[22]如下：

（3）

其中R是空气中的气体常数，值为287.052 87 ；为大气海平面高度的空气密度，值为1.225 ；为大气海平面高度的温度，值为288.15 K/15℃；是对流层顶部以下的温度梯度，值为-0.006 5 ；g是重力加速度，值为9.8 ；为地点标高；为无人机飞行的高度。

将无人机运行分为3个阶段：水平飞行阶段、起升阶段和下降阶段。不同飞行阶段的能耗不同。当无人机处于起飞、下降阶段时，推力主要用于克服自身重力。无人机的运行能耗[23]为：

（4）

（5）

（6）

（7）

飞行过程中的空气密度为[24]：

（8）

式中：，和分别表示起飞、降落和水平运行阶段的能耗；表示运行过程的总能耗；表示水平飞行距离；，，分别表示螺旋桨工作效率、电机工作效率和电调工作效率；为无人机总质量；为修正因子；，，分别表示水平、垂直向下和垂直向上速度分量。为迎风面积；为零阻力升力系数；诱导阻力因子；为螺旋桨桨盘面积；表示当前路径点高度与下一路径点高度处空气密度的平均值。

3.3 路径平滑

ECTG可以生成一系列路径点，通过将这些路径点按顺序连接起来，得到一条避开障碍物的路径。然而，该路径不能满足连续飞行的实际要求。因此，使用三次B样条曲线[25]对路径进行平滑处理。将生成的路径点作为B样条基函数的控制点以生成具有连续曲率的光滑路径。假设生成的路径点依次为：（N1，N2，…，Nn），三次B样条曲线的基函数如下式所示[10]：

（9）

Ni和Ni+3间的三次B样条曲线可以表示为：

（10）

4 算例分析

4.1 参数设置

为了验证所提出算法的有效性，本研究设计了多维度对比实验：首先，在实验中选取4种具有代表性的路径规划算法作为对比基准，包括基于图搜索的A*算法、基于抽样的RRT算法、基于群智能体的PSO算法和基于矢量场的VFH算法，这4种算法分别代表了不同的路径规划策略；其次，构建了3种典型的障碍分布场景(分别标记为S1，S2和S3)，具体布局如图4所示。这些场景根据障碍物的密度和重叠程度而有所不同：S1为低密度且障碍物为圆形的场景、S2为包含椭圆形和圆形障碍物的低密度场景、S3为高密度复杂障碍物场景。通过这种方式，可以全面评估不同算法在各种环境下的表现。

实验分别在二维平面与三维空间环境中进行，对各算法的路径长度、转向次数和能量消耗等指标进行定量分析。该设计通过算法类型与场景复杂度的正交组合，旨在验证所提方法在不同空间维度下的综合性能优势。根据文献[26]和[27]的相关数据以计算能量消耗，无人机的相关参数如表2所示。

图4 障碍物分布场景

表2无人机的参数

| 模型参数 | 数值 | 模型参数 | 数值 | 模型参数 | 数值 |
| --- | --- | --- | --- | --- | --- |
|  | 0.8 |  | 0.94 |  | 15.000 |
|  | 0.8 |  | 0.75 |  | 1.313 |
|  | 0.8 |  | 0.13 |  | 0.015 |

4.2 仿真实验结果对比

首先，验证改进的椭圆切线图法在二维路径规划中的有效性。在3种障碍物分布场景下将其分别与椭圆切线图法、A*算法、RRT算法、PSO算法和VFH算法进行对比分析，通过分析路径长度与转弯次数以评估算法性能。路径规划结果可视化如图5所示，数据汇总如表3所示。

通过对图5和表2的数据分析可以发现：无论障碍物分布如何变化，改进的椭圆切线图法均能生成最短且转向次数最少的路径，且环境越复杂，其优势越明显。这是由于该方法考虑了子路径的长度和避开障碍物时所生成的切线路径，提升了路径规划的质量；RRT算法是基于随机抽样的方法生成路径点，没有提高路径质量的机制，因此难以保证路径的最优性，容易导致无人机频繁转向和绕行；相比之下，A*算法作为一种基于网格的确定性搜索算法，能够生成从起点到终点的可行路径但其生成的路径的质量不是优秀的；VFH算法是一种局部路径规划算法，搜索范围有限，关注于避开前方可见的障碍物，忽视了整体的路径长度。

其次，验证ECTG在三维路径规划中的有效性。随机设置3种障碍物布局中的障碍物高度，且障碍物的高度被设定在低空可行区域范围内（即20-120 m），无人机飞行速度设置为15 m/s。将ECTG分别与三维A*算法、RRT算法、PSO算法和VFH算法进行对比分析，通过分析路径长度、转弯次数和能量消耗以评估算法性能。路径规划结果可视化如图6所示，数据汇总如表4所示。

图5 二维路径规划结果

表3 二维路径规划算法结果比较

|  |  | 改进的椭圆切线图法 | 椭圆切线图法 | A* | RRT | PSO | VFH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 路径长度/m | S1 | 147.0 | 147.6 | 160.6 | 172.4 | 179.1 | 152.3 |
|  | S2 | 134.9 | 136.0 | 144.9 | 169.9 | 185.1 | 147.5 |
|  | S3 | 284.2 | 288.4 | 298.0 | 329.6 | 331.3 | 330.2 |
| 能量消耗/J | S1 | 8 311.52 | 8 345.44 | 9 080.48 | 9 747.66 | 10 126.48 | 8 611.19 |
|  | S2 | 7 591.84 | 7 563.75 | 8 154.62 | 9 561.55 | 10 416.97 | 8 300.94 |
|  | S3 | 15 992.54 | 16 228.88 | 16 769.10 | 18 547.29 | 18 642.96 | 18 581.06 |
| 转向次数/次 | S1 | 3 | 3 | 5 | 12 | 2 | 7 |
|  | S2 | 1 | 2 | 5 | 14 | 3 | 9 |
|  | S3 | 2 | 4 | 9 | 18 | 5 | 10 |

表4 三维路径规划算法结果比较

|  |  | ECTG | A* | RRT | PSO | VFH |
| --- | --- | --- | --- | --- | --- | --- |
| 路径长度/m | S1 | 149.2 | 175.9 | 190.7 | 185.7 | 158.0 |
|  | S2 | 136.2 | 149.0 | 190.5 | 167.2 | 146.4 |
|  | S3 | 284.9 | 300.9 | 320.5 | 306.0 | 290.5 |
| 能量消耗/J | S1 | 12 482.46 | 21 074.25 | 21 951.52 | 35 326.29 | 29 265.42 |
|  | S2 | 12 978.31 | 19 991.08 | 20 623.59 | 31 403.52 | 27 099.29 |
|  | S3 | 25 653.76 | 64 610.08 | 34 848.73 | 58 222.90 | 58 098.41 |
| 转向次数/次 | S1 | 2 | 6 | 8 | 9 | 5 |
|  | S2 | 1 | 5 | 10 | 10 | 5 |
|  | S3 | 2 | 5 | 10 | 10 | 10 |

图6 三维路径规划结果

从图6的路径规划结果可以看出，ECTG、A*算法、RRT算法、PSO算法及VFH算法均可成功规划出一条无人机从起点出发、避开障碍物并最终顺利到达终点的无碰撞路径。再结合表3的数据发现：本文提出的ECTG算法在路径长度、转向次数以及能量消耗等关键指标上都全面优于其它4种路径规划算法，特别是在能量消耗方面表现优越，且随着环境复杂度的增加，其优势更加显著；此外，A*算法在减少转向次数方面较为出色、VFH算法在缩短路径长度方面较优、RRT算法由于其随机抽样的特点导致其在路径长度和转向次数方面表现较差；结合具体数据分析可以发现，ECTG算法在三种场景中与A*算法、RRT算法、PSO算法和VFH算法相比，平均路径长度分别减少了8.7 %、18.7 %、13.4 %、4.1 %，平均转向次数分别减少了68.8 %、82.1 %、82.8 %、75.0 %，平均能量消耗分别减少了51.6 %、34.0 %、59.1 %、55.3 %。综上所述，这些实验对比结果充分验证了ECTG算法的有效性和先进性，特别是在处理复杂三维环境时，ECTG不仅能够提供更短、更平滑的飞行路径，还能显著降低无人机的能量消耗，从而延长任务执行时间和提高操作效率。

5 结论

（1）基于无人机的安全飞行约束，提出考虑无人机最小安全距离的障碍物建模方法，在降低计算复杂度的同时消除了传统包络方法导致的路径冗余。

（2）针对二维路径规划方法，采用双向搜索策略和节点筛选机制对传统的椭圆切线图法进行改进。结果表明，改进后的方法可以克服原方法在路径长度和频繁转向上的不足，且与A*算法，RRT算法、PSO算法和VFH算法相比展现出更优的综合优势。

（3）提出一种基于切线图并考虑能耗的三维路径规划方法，相较于A*、RRT、PSO和VFH算法，其平均路径长度分别减少8.7 %、18.7 %、13.4 %和4.1 %，转向次数降低了68.8-82.8 %，能量消耗缩减了34.0-59.1 %，缩短了路径长度，提高了无人机路径的平滑性，增强了无人机路径避障的安全性，突破了传统方法在路径经济性、运动平滑性与能量效率间的权衡瓶颈，为复杂环境下的无人机自主导航提供了兼具安全性和最优性的解决方案。

参考文献（References）

- KHAN M T R, MUHAMMAD SAAD M, RU Y, et al. Aspects of unmanned aerial vehicles path planning: overview and applications[J]. International Journal of Communication Systems, 2021, 34(10): e4827.

- YIN C, XIAO Z, CAO X, et al. Offline and online search: UAV multi-objective path planning under dynamic urban environment[J]. IEEE Internet Of Things Journal, 2018, 5(2): 546–558.

- SU W, GAO M, GAO X, et al. Enhanced multi-UAV path planning in complex environments with voronoi-based obstacle modelling and q-learning[J]. International Journal of Aerospace Engineering, 2024, 2024: 1–14.

- SU W, GAO M, GAO X, et al. Enhanced multi-UAV path planning in complex environments with voronoi-based obstacle modelling and q-learning[J]. International Journal of Aerospace Engineering, 2024, 2024: 1–14.

- CAO Z, LI D, ZHANG B. Dynamic trajectory planning for UAV cluster by weighted voronoi diagram with particle swarm optimization[C]. FU W, GU M, NIU Y, eds.//Proceedings of 2022 International Conference on Autonomous Unmanned Systems (ICAUS 2022). Singapore: Springer Nature, 2023: 3479–3490.

- 姚千,杨洁.改进A*算法在三维空间中无人机的航迹规划[J].传感器与微系统,2025,44(3):143-147.

YAO Q, YANG J. Improved A*algorithm for UAV trajectory planning in 3D space[J]. Transducer and Microsystem Technologies, 2025, 44(3): 143-147. (in Chinese)

- 唐宇洋, 郑恩辉, 邱潇, 等. 基于优化双向A*与人工势场法的无人机三维航迹规划. 空军工程大学学报 2024, 25 (5), 69–75.

TANG Y Y, ZHENG E H, QIU X, et al. 3D UAV Trajectory Planning Based on Optimized Bidirectional A and Artificial Potential Field Method[J]. Journal of Air Force Engineering University, 2024, 25 (5), 69–75. (in Chinese)

- H. Rohnert, Shortest paths in the plane with convex polygonal obstacles[J]. Information Processing Letters, 1986, 23(2): 71-76.

- CHEN H, CHANG K, AGATE C S, et al. UAV path planning with tangent-plus-lyapunov vector field guidance and obstacle avoidance[J]. IEEE Transactions on Aerospace and Electronic Systems, 2013, 49(2): 840–856.

- LIU H, LI X, FAN M, et al. An autonomous path planning method for unmanned aerial vehicle based on a tangent intersection and target guidance strategy[J]. IEEE Transactions On Intelligent Transportation Systems, 2022, 23(4).

- LI W, WANG L, ZOU A, et al. Path planning for UAV based on improved PRM[J]. Energies, 2022, 15(19): 7267.

- 姚信宇, 王燮, 王程,等. 基于改进Goal-bias RRT的无人机三维路径规划研究[J].飞行力学, 2024: 1–8.

YAO X Y, WANG X, WANG C, et al. Research on UAV3D path planning based on improved Goal-bias RRT[J]. Flight Dynamics, 2024: 1–8. (in Chinese)

- HAO G, LV Q, HUANG Z, et al. UAV path planning based on improved artificial potential field method[J]. Aerospace, 2023, 10(6): 562.

- BORENSTEIN J, KOREN Y. The vector field histogram-fast obstacle avoidance for mobile robots[J]. IEEE Transactions on Robotics and Automation, 1991, 7(3): 278–288.

- Ding K, Chen M, Yong K. Self-adjusting angular resolution-based obstacle avoidance for inspection unmanned aerial vehicles in nuclear power stations[C/OL]//2022 41st Chinese Control Conference (CCC). [2025–04–07].

- Li W, Cheng L, Hu J. Research on stealthy UAV path planning based on improved genetic algorithm[C/OL]//2022 International Conference on Artificial Intelligence and Computer Information Technology (AICIT). [2025–04–03].

- Huan L, Ning Z, Qiang L. UAV path planning based on an improved ant colony algorithm[C/OL]//2021 4th International Conference on Intelligent Autonomous Systems (ICoIAS). [2025-04-03].

- DENG L, CHEN H, ZHANG X, et al. Three-dimensional path planning of UAV based on improved particle swarm optimization[J]. Mathematics, 2023, 11(9): 1987.

- AGGARWAL S, KUMAR N. Path planning techniques for unmanned aerial vehicles: a review, solutions, and challenges[J]. Computer Communications, 2020, 149: 270–299.

- YANG K, SUKKARIEH S. Real-time continuous curvature path planning of UAV in cluttered environments[C/OL]//2008 5th International Symposium on Mechatronics and Its Applications. [2025–04–10].

- MARDANI A, CHIABERGE M, GIACCONE P, et al. Communication-aware UAV path planning[J]. IEEE Access, 2019, 7: 52609–52621.

- GONG H, HUANG B, JIA B, et al. Modeling power consumptions for multirotor uavs[J]. IEEE Transactions on Aerospace and Electronic Systems, 2023, 59(6): 7409–7422.

- LI Y, LIU M. Path planning of electric VTOL UAV considering minimum energy consumption in urban areas[J]. Sustainability, 2022, 14(20): 13421.

- AIELLO G, VALAVANIS K P, RIZZO A, et al. Fixed-wing UAV energy efficient 3d path planning in cluttered environments[J]. Journal of Intelligent & Robotic Systems, 2022, 105(3): 60.

- Elbanhawi M, Simic M, Jazar R N, et al. Continuous path smoothing for car-like robots using b-spline curves[J]. Journal Of Intelligent & Robotic Systems, 2015, 80: S23–S56.

- 赵嶷飞,郑雨欣.城市物流无人机飞行任务剖面构建与优化[J].飞行力学,2021,39(3):54-59.

ZHAO N F, ZHEN Y X. Construction and optimization of flight mission profile of urban logistics UAV[J]. Flight Dynamics, 2021,39(3):54-59. (in Chinese)

- D’Andrea R. Guest editorial can drones deliver? [J]. IEEE Transactions on Automation Science and Engineering, 2014, 11(3): 647–648.

Three-dimensional path planning for UAVs considering flight energy consumption: an approach based on improved elliptic tangent maps

Abstract: With the development of UAV technology, UAVs are increasingly used in many fields such as logistics, agriculture, and environmental monitoring. However, three-dimensional path planning for UAVs faces the challenges of insufficient optimization of energy consumption and traditional methods in terms of path smoothness and safety. To this end, this paper proposes a 3D path planning method for UAVs based on tangent graphs and considering energy consumption. First, in order to meet the safe flight requirements of UAVs, an ellipsoidal model is used to model irregular obstacles with a safety envelope, and an obstacle modeling method is proposed to consider the minimum safe distance of UAVs. The method eliminates the path redundancy caused by the traditional envelope method while reducing the computational complexity, and improves the planning efficiency and safety. Second, the traditional elliptic tangent graph method is improved by introducing a two-way search strategy and a node screening mechanism to generate two-dimensional reference path points, which effectively reduces the number of path steering and shortens the path length. Finally, the 2D baseline path points and energy consumption model are coupled to generate the UAV path planning method in 3D scenarios. The 3D reference path points are constructed based on the 2D reference path points, and when the starting point and the end point are at the same altitude, the 3D planning problem is transformed into a 2D planar problem by using the downscaling strategy, and the improved elliptic tangent map method is utilized to solve the problem. For the start and end points with height difference, an energy consumption evaluation model is established to compare the energy consumption of the two strategies of horizontal fly-by and climb-through, and output the path point with the lowest energy consumption, and use the generated path point as the control point of the B-spline basis function to generate a smooth path with continuous curvature. In order to verify the effectiveness of the method, three test scenarios with different obstacle densities and layouts are constructed, and compared with four existing typical representative algorithms (A* algorithm, RRT algorithm, PSO algorithm, and VFH algorithm), and the results show that, in a two-dimensional scenario, regardless of changes in the distribution of the obstacles, the improved elliptic tangent map method is able to generate the shortest paths with the least number of steering times and its advantages are more obvious the more complicated the environment is. The more complicated the environment is, the more obvious the advantage is. In complex 3D environments, this method not only provides shorter and smoother flight paths, but also significantly reduces the energy consumption of the UAV. Specifically, the average path length is reduced by 8.7%, 18.7%, 13.4%, and 4.1%, the average number of turns is reduced by 68.8%, 82.1%, 82.8%, and 75.0%, and the average energy consumption is reduced by 51.6%, 34.0%, 59.1%, and 55.3%, respectively. The trade-off between path length, motion smoothness and energy consumption in the traditional method is effectively solved, which provides a solution with both theoretical innovation and engineering practicability to enhance the mission endurance of UAVs

Key words：UAV; 3D path planning; Tangent map; Energy consumption; Obstacle avoidance