基于实时反演和变形演化的隧洞结构安全评价

摘 要：引水隧洞在长期服役过程中受物理化学耦合作用导致结构性能劣化。针对现有安全评价方法多依赖定性指标和经验判据，难以实现结构安全状态量化评估的问题，提出了一种基于实时反演和变形演化的结构安全评价新方法。该方法首先对隧洞原始变形监测序列进行数据预处理，获取高质量的时效变形数据；然后采用基于深度学习和贝叶斯优化的物理信息引导的反演方法获取当前隧洞力学参数。基于内变量热力学理论下的弹-黏塑性-损伤蠕变本构模型和对材料的蠕变行为特征分析，构建了包含长期变形加速安全系数(S1)、变形非线性起始安全系数(S2)和短期变形加速安全系数(S3)的隧洞“3S”结构安全量化评价时效指标体系，并阐释了各指标的物理意义。为模拟材料劣化效应，通过设定不同降强系数表征材料性能的渐进衰减，开展多工况数值模拟以揭示隧洞结构随劣化程度加剧的变形演化规律。将该分析评价框架应用于JLL隧洞工程，得到S1为2.4~3.0，S2为3.7~4.6，S3为5.7~7.3，量化了不同劣化程度对应的结构安全临界状态，验证了该方法的可行性。

关键词：隧洞；安全评价；力学参数反演；变形演化；材料劣化

中图分类号：TV672+.1    文献标志码：A

随着水利水电工程建设的持续推进，作为输水系统核心组成部分的引水隧洞规模日益扩大。然而，在复杂的运行环境中，隧洞结构长期经受物理化学的耦合作用，导致微观损伤持续演化，其力学性能不可避免地发生退化。这种渐进性的结构损伤与材料劣化的累计最终可能诱发隧洞失稳乃至垮塌风险，直接威胁工程效益与区域安全[1]。因此，建立科学有效的隧洞结构安全评价体系，全面掌握引水隧洞的运行状态，对延长服役寿命与预防潜在灾害具有重要的工程意义。

对于隧洞安全状态，各国家采取了多种评价标准和体系。美国相关规范[2]关注衬砌裂缝、衬砌剥落和渗漏水3方面，以此衡量隧洞结构安全。日本相关标准[3]除了考量材料劣化和渗漏水，还将外荷载等方面纳入隧洞结构风险因素。中国相关规范标准[4-7]综合考虑衬砌裂缝、衬砌强度、流速和压力等多个方面评估隧洞结构安全，但目前的评价方式以定性评价为主，难以准确量化隧洞结构安全状态。

为解决定性评价的局限性，实现更精确的隧洞安全状态评估，近年来相关研究提出了多种基于特定指标或理论的评价方法。靳春玲[8]基于PSR模型选取了混凝土剥落、地表水资源量和防灾应急能力等31个指标评价引水隧洞安全。汤雷等[9]聚焦隧洞衬砌结构，提出了包含混凝土强度、碳化深度、保护层厚度与钢筋锈蚀状态四个方面的安全分级评价体系。贺思乐等[10]选取岩石质量指标、湿抗压强度、完整性系数、结构面强度系数、地下水渗水量5个因素作为隧洞结构安全评价指标。辛萌等[11]基于材料强度、结构配筋和承载力安全系数衡量隧洞结构安全。陆丽丽等[12]选取材料劣化、衬砌裂缝、渗漏水和衬砌剥落4个指标度量隧洞安全状态，采用改进的G2-反熵权法计算指标权重。刘万林等[13]选取19个二级指标，基于改进的G2-AEW-UMT模型构建水工隧洞安全风险评价指标体系。张翌娜等[14]从结构响应、耐久性、外界因素等方面对TBM引水隧洞结构安全进行综合评价。上述研究侧重于建立涵盖隧洞安全多维度影响因素的指标体系，并探索科学合理的权重确定方法。

随着概率统计与机器学习等理论的发展，智能算法与数据驱动方法被用以构建隧洞安全评估模型。祁英弟等[15]引入灰色关联理论，建立中国西北地区引水隧洞安全评价模型。Zhang等[16]融合改进的D-S证据理论与BP神经网络算法，实现了对隧洞安全状态的智能诊断。刘康等[17]综合文献与专家意见，构建了用于引水隧洞安全状态评估的动态贝叶斯网络模型。Liu[18]整合可能性理论与前景理论，提出了一种隧道安全状态评估新方法。Zhao等[19]聚焦软岩蠕变条件下的隧道结构力学响应，引入知识决策与数据驱动相结合的混合评估框架，对隧洞结构长期安全性能进行分析。Liu等[20]设计了一种基于半监督学习与堆叠集成算法的隧道安全综合评估方法。Chen等[21]提出了结合Pair-Copula函数和贝叶斯网络的盾构隧道风险评估模型，识别了变形和沉降风险的安全风险类别。Jin等[22]提取裂缝长度、深度、体积、面积、类型及位置等多维度特征，应用多种机器学习算法对隧洞衬砌裂缝的安全性进行判别。Liu等[23]聚焦硫酸盐侵蚀后隧道衬砌的病害特征，提出一种基于微量元素分层法的衬砌安全评价方法。

上述研究在隧洞安全评价方法创新与体系构建方面取得了显著进展，然而在工程实践层面仍面临诸多挑战。一方面，现有指标的评判标准及权重分配多依赖于专家经验，评价结果易受个体认知偏差影响，评价体系具有一定的主观依赖性。另一方面，部分评价指标体系包含过多参数，数据获取成本高且操作复杂，制约了其在工程中的推广应用。此外，多数方法未充分考虑隧洞结构在长期服役过程中的渐进性损伤演化特征，难以动态反映结构安全状态演变。

为此，本文提出了一种基于实测时效变形数据的隧洞结构安全量化评价方法。通过数据预处理后的位移反演获取当前隧洞力学参数。采用基于内变量热力学的蠕变损伤本构，考虑隧洞在长期服役过程中的损伤劣化，开展不同降强工况下的数值仿真计算。提出隧洞3S结构安全时效评价指标，通过分析不同劣化程度下隧洞变形演化曲线序列，对隧洞结构安全进行量化评价，为工程安全运维提供参考和指导。

1 隧洞运行期结构安全评价框架

提出隧洞运行期结构安全量化评价框架如图1所示。整个评价流程可划分为以下4个阶段：

（1）获取隧洞原始变形监测数据，并进行数据预处理，包含数据插补、数据修正和数据降噪操作，得到满足后续反演要求的隧洞时效变形数据。

（2）采用基于深度学习和贝叶斯优化的隧洞力学参数反演方法，开展力学参数反演，获取当前隧洞力学参数，作为后续数值仿真计算的基准值。

（3）考虑隧洞结构在长期服役过程中的物理化学损伤效应，将其概化为隧洞力学参数的降低。采用基于内变量热力学理论的蠕变损伤本构模型，通过引入损伤参数和不同程度的降强系数，模拟不同劣化条件下隧洞结构的变形演化过程。

（4）通过不同劣化程度下的时效计算，获取隧洞变形演化曲线序列。基于所提3S结构安全系数评价指标，获取隧洞结构安全量化评价结果。

图1  隧洞结构安全量化评价框架

2 基于变形演化的隧洞结构安全评价指标

2.1  弹-黏塑性蠕变损伤本构模型

为准确评估隧洞在长期运行状态下的稳定性和安全性，隧洞结构安全演化分析采用弹-黏塑性蠕变损伤本构模型[24-26]。其本构方程一般形式如下：

,                (1)

,                (2)

.       (3)

其中：, 和 分别为应变率、弹性应变率和黏塑性应变率，为应力张量，为四阶柔性张量， 是关于和内变量的函数。

与黏塑性内变量共轭的热力学力为：

,                (4)

.          (5)

其中：a、b为材料参数，为应力张量第一不变量，为应力偏量第二不变量。

与共轭的热力学力为

.             (6)

黏塑性应变率方程为：

,          (7)

.     (8)

其中：为黏塑性体应变，为黏塑性偏应变分量，为应力偏量分量。

内变量演化由热力学力驱动且存在阈值，演化方程为：

,             (9)

,             (10)

.      (11)

其中：、、均为黏滞系数，m为方程参数，p、h、R为材料常数。

2.2  材料蠕变特征分析

该弹-黏塑性蠕变损伤本构模型通过3种宏观内变量的演化方程及其特性，能够表征岩石或混凝土材料的过渡蠕变、稳态蠕变和加速蠕变3个阶段，如图2所示：

1）内变量的演化方程（式(9)）属于硬化型方程。随时间增加，值逐渐增大但其变化率迅速衰减并趋近于零，因此实际控制过渡蠕变阶段（AB段）；

2）当应力超越屈服面并保持恒定时，的变化率基本保持不变（式(10)），故主导稳态蠕变阶段（BC段）；

3）内变量的演化方程（式(11)）呈指数函数形式，其变化率随自身值增大持续提升。当演化至特定程度时，其数值将呈指数级增长，导致黏塑性应变率急剧增大，因此控制加速蠕变阶段（CD段）。

综上分析可知，本研究采用的本构模型能够有效刻画图2中岩石或混凝土材料在不同应力水平或力学参数强弱状态下的时效变形行为：

1）低应力水平或强参数状态时，材料仅经历过渡蠕变阶段，此时应力状态位于屈服面内，蠕变速率逐渐趋零并最终达到平衡（模式I）；

2）中等应力水平或中等参数状态时，应力超越屈服面，材料在过渡蠕变后进入稳态蠕变阶段，蠕变速率保持基本恒定。在黏塑性变形演化过程中，内部损伤与微观结构破坏累积引发加速蠕变（模式II）；

3）高应力水平或弱参数状态时，材料内部损伤与微观结构破坏快速发展，短期内即进入加速蠕变阶段（模式III）。

图2  材料经典蠕变曲线

2.3  基于变形演化的隧洞结构安全评价指标

由上述分析可知，当应力水平一定时，不同强弱状态的力学参数，会诱发材料不同的时效变形行为。考虑隧洞在长期服役过程中的结构损伤和材料劣化，将物理-化学-时间效应对隧洞结构安全的影响概化为力学参数的弱化。基于此，提出隧洞3S结构安全评价指标。

为表征力学参数的劣化程度，定义降强系数Kr：

(12)

其中：Pr为劣化工况下隧洞的力学参数，P0为隧洞当前实际力学参数，由反演得到。

结构安全系数S定义为

(13)

隧洞3S结构安全系数包含长期变形加速结构安全系数S1、变形非线性起始结构安全系数S2和短期变形加速结构安全系数S3，如图3所示，可分别由式(14)-(16)计算得到：

(14)

(15)

(16)

其中：Krl为隧洞随着力学参数的不断降低首次出现加速蠕变阶段对应的降强系数，Kri为隧洞随着力学参数的不断降低初始变形速率开始呈现非线性时对应的降强系数，判断标准为初始变形速率与降强系数的皮尔逊相关系数小于0.975。Krs为隧洞在定义的短期Ts时间内（本研究中取Ts=3 000 h）随着力学参数的不断降低首次出现变形明显加速对应的降强系数。

各安全系数的物理意义如下：

（1）长期变形加速结构安全系数S1：表征隧洞结构在力学参数持续衰减条件下，长期抵抗显著蠕变失稳的能力，揭示了隧洞结构在经历逐级劣化后，其长期变形行为发生根本性转变的临界点。此时，隧洞变形速率将不再趋于稳定或缓慢增加，而是进入加速蠕变阶段，预示着结构可能因长期流变效应而最终丧失承载能力。S1是衡量隧洞长期稳定性裕度的关键指标，数值越高表明结构抵抗长期蠕变破坏的能力越强。

（2）变形非线性起始结构安全系数S2：表征隧洞结构在力学参数折减过程中，其初始变形响应偏离线性状态的临界点，反映了隧洞在变形演化初期，材料非线性力学行为开始主导变形机制的起始状态。S2 作为揭示隧洞结构进入非线性工作状态的指标，数值越高表明结构在较大强度损失下才表现出显著非线性，其安全储备越高。

（3）短期变形加速结构安全系数S3：表征隧洞结构随力学参数的不断弱化，在短期内抵抗显著变形失稳的能力。S3反映了在定义的Ts时间内，结构变形由相对稳定状态突然转向加速发展的临界劣化状态，预示着在该参数状态下，结构可能临近坍塌或局部失稳的阈值。S3数值越高，表明结构在力学参数弱化后维持短期稳定性的能力越强，突发失稳风险越低。

S1、S2和S3分别从长期时效变形发展、材料非线性行为起始和短期失稳风险三个物理层面，刻画了隧洞结构在力学参数劣化路径上的不同安全状态，能从总体上评价隧洞的结构安全状态，为隧洞工程的全生命周期安全评价与预警提供了量化依据。

图3  基于变形演化的隧洞3S结构安全评价指标

3 基于监测数据的力学参数实时反演方法

3.1  监测数据预处理

由于传感器故障、人工操作失误或外界因素干扰，原始隧洞监测数据可能存在数据缺失、异常和噪声等问题，这直接影响了力学参数反演的准确性。因此，在基于实测变形数据进行反演前，需要对原始数据进行预处理，包括填补缺失值、识别并修正异常值、数据平滑处理以削弱噪声等流程。

对于存在缺失值的变形时间序列，采用K近邻法（K-Nearest Neighbors, KNN）进行数据插补。KNN是一种基于实例的非参数学习算法，被广泛应用于分类、回归和数据插补等任务[27-29]。该算法的核心思想在于利用数据样本间的相似性。对于包含缺失值的特定时间点t的目标序列，KNN首先在整个数据集或子集中寻找与其在完整维度上最相似的K个序列，并采用欧式距离量化相似性。在确定K个最近邻序列后，目标序列在时间点t的缺失值则通过加权平均这些近邻序列在对应时间点t的观测值进行估计。

为识别并修正原始变形监测数据中的异常值，采用一种基于Z分数的局部均值替代法。Z分数用于量化观测值偏离其均值的标准差倍数。采用滑动窗口策略计算Z分数，异常值经识别后，利用其相邻非异常值的算术平均值进行替代。该方法在有效抑制极端值干扰的同时，也保留了原始数据的整体趋势特征。

在前述数据处理的基础上，基于隧洞变形发展规律，使用对数趋势拟合对数据进行平滑处理以削弱噪声。

图4展示了数据预处理前后的时效变形数据对比。可以看到，上述数据预处理方法有效填补了缺失值，修正了异常值，同时削弱了数据噪声。

图4  数据预处理前后的变形曲线对比

3.2  力学参数实时反演

基于数据预处理后的实测变形时间序列，开展隧洞力学参数的实时反演。本研究采用一种基于深度学习和贝叶斯优化的物理信息引导的隧洞力学参数反演框架[30]，其实现流程如图5所示。通过全连接层(Fully Connected Layer, FCL)和门控循环单元(Gated Recurrent Unit, GRU)建立深度学习模型，将数据损失与物理损失相结合，构建物理信息引导的隧洞代理模型框架。经超参数优化，训练得到最优的隧洞代理模型。

图5  隧洞力学参数反演流程

为避免预测的力学参数符合实测变形数据但却脱离真实的参数范围的情况，首先需要定义力学参数的取值范围，以确保优化过程中所考虑的参数值在物理意义上是合理的。同时，考虑力学参数之间的物理关联，通过在目标函数中添加惩罚项来引导优化过程满足参数之间的相关性。

贝叶斯优化算法初始在参数空间中随机猜测一组力学参数，将其代入使用最优超参数训练好的隧洞代理模型中，获取预测的变形时间序列。然后结合实测变形时间序列，计算目标函数值，通过高斯过程回归更新先验分布，并基于获取函数获取下一个采样点，不断迭代优化。当达到设定的最大迭代次数时，输出使目标函数值最小的最优力学参数。

4 工程应用

4.1  工程概况与数值模型

将上述方法应用于湖南九龙岭隧洞的结构安全评价中。隧洞总长20 km，地质条件复杂。根据隧洞中40 m洞段的实际尺寸建立了数值模型和计算网格，如图6所示。隧洞断面形态为圆拱直墙式，圆拱半径为3.2 m，圆心角为120°，直墙高度为4.1 m。计算网格的横向长度为100 m×100 m（x×z），纵向长度为40 m（y）。为提高隧道开挖边界附近的计算精度，对隧道轴线附近 10 m范围内的计算网格进行了加密处理。选取计算模型y=20 m处的隧洞断面作为监测断面，依据工程的实际监测点布置，设置拱顶（A）、左右拱肩（B、C）和左右拱腰（D、E）共5个监测点，以记录该断面各关键部位的时效变形。

图6  计算网格与监测点布置

4.2  参数反演与劣化计算

采用第3节的反演方法，基于预处理后的实测变形获取围岩力学参数，相应的变形对比曲线如图7所示，可以看到，反演值与实测值吻合良好。衬砌变形参数参考设计取值，黏塑性参数参考文献[30]中II类岩力学参数反演值进行近似取值，各力学参数初始值如表1所示。考虑隧洞劣化的不同程度，设置不同的计算工况，对应0.1~1范围的不同降强系数。其中，Kr=1代表不考虑材料劣化的正常工况。计算隧洞在不同劣化工况下的变形演化过程。

图7  力学参数反演结果对比

表1  隧洞初始力学参数

| 参数类型 | 参数 | 围岩 | 衬砌 |
| --- | --- | --- | --- |
| 变形参数 | K (GPa) | 6.67 | 13.89 |
|  | G (GPa) | 4.13 | 10.42 |
| 黏塑性参数 | ηp1 (GPa∙s) | 1.75×106 | 2.48×106 |
|  | h (GPa) | 12.42 | 15.18 |
|  | α | 0.31 | 0.34 |
|  | R (MPa) | 1.05 | 1.52 |
|  | σt (MPa) | 2.19 | 4.25 |
|  | κp2 (s-1) | 5.63×10-10 | 3.22×10-10 |

4.3  隧洞结构安全量化评价

隧洞各特征监测点在不同Kr下的均值变形曲线如图8所示。可以看到，在不同劣化程度下，隧洞时效变形呈现不同的演化特征。正常工况下，变形在演化1 500 h左右开始收敛并最终趋于稳定。当Kr为0.7~0.9时，隧洞初始变形速率略有增大，但仍趋于稳定，同时变形收敛值随着劣化系数的降低而增大。当Kr为0.6时，隧洞变形先迅速增大再缓慢增长，经过一段时间后加速增长。当Kr为0.3时，隧洞在演化3 000 h时出现显著加速变形阶段。随着Kr进一步降低，隧洞的初期变形速率和变形量值明显增大，同时出现加速变形的时刻不断提前。

可见，对于该隧洞而言，当Kr≥0.7，对应劣化程度轻微，此时隧洞在初期变形收敛后基本可以保持长期变形稳定，表明此状态下隧洞结构具有长期安全性。当Kr在0.4~0.6之间，对应劣化程度中等，此时隧洞在短期内可以保持变形缓慢增长，但随着时间的延长，会出现加速变形阶段，表明该工况下隧洞结构具有短期安全性，但不具备长期安全性。当Kr≤0.3时，对应劣化程度严重，此时隧洞在短期内会出现加速变形阶段，表明该工况下隧洞在短期内存在结构失稳风险，隧洞不具备短期安全性。

图8  不同降强工况下隧洞时效变形曲线

隧洞初始变形速率及皮尔逊相关系数随Kr变化曲线如图9所示。可以看到，随着Kr的不断降低，初始变形速率呈现先线性增大后逐渐进入非线性并显著增长的变化规律。当Kr为0.4~0.5时，变形速率恰好由线性增大转变为非线性增长。

基于2.3节的定义与计算结果，得到隧洞3S结构安全系数如表2所示。其中，长期变形加速结构安全系数S1为2.4~3.0，变形非线性起始结构安全系数S2为3.7~4.6，短期变形加速结构安全系数S3为5.7~7.3，表明当前隧洞结构是安全的，且具有一定安全裕度。

与现有的其他隧洞安全评价指标相比，所提3S指标更加精简，同时充分利用隧洞实测变形数据且不局限于隧洞当前状态信息，而侧重于从演化的角度，考虑损伤劣化，评价隧洞的未来结构安全。

图9  初始变形速率及皮尔逊相关系数-Kr曲线

表2  隧洞结构安全系数

| 结构安全系数 | S1 | S2 | S3 |
| --- | --- | --- | --- |
| 取值范围 | 2.4~3.0 | 3.7~4.6 | 5.7~7.3 |

5 结论

针对目前难以定量评价隧洞结构时效安全的问题，本文提出了一种隧洞结构安全量化评价框架和指标，并应用于隧洞工程。得出主要结论如下：

（1）采用了物理信息引导的隧洞力学参数实时反演方法，针对原始监测数据可能存在的缺失、异常和噪声问题，分别采取KNN算法、滑窗Z分数替代法和对数趋势平滑法进行处理，有效提高了数据质量，为隧洞安全评价提供数据基础。

（2）提出隧洞3S结构安全系数评价法，采用长期变形加速系数、变形非线性起始系数和短期变形加速系数，量化不同劣化程度下隧洞结构安全状态。该方法结合变形速率非线性拐点与加速阶段临界条件，为隧洞短期与长期安全状态的分级预警提供了依据。

（3）构建了隧洞结构安全量化评价框架。首先对原始变形监测数据进行预处理，再基于时效变形数据反演隧洞力学参数；通过设置不同的降强工况，模拟不同劣化条件下的隧洞变形演化；最终采用所提结构安全系数评价指标，实现对隧洞结构安全性的量化评价。

（4）将所提方法应用于隧洞工程，得出3S结构安全系数。当降强系数为0.7~1.0时，隧洞变形呈现先增大后逐渐趋于稳定的规律。当降强系数为0.4~0.6时，隧洞变形呈现先迅速增大再逐渐趋缓最后加速上升的规律。当降强系数为0.1~0.3时，随着降强系数的降低，隧洞初始变形速率显著增大，同时出现加速变形的时间不断提前。

本文所提方法是基于时效仿真分析的，变形演化特征与结构安全相对应，因此，对隧洞长期安全稳定性评价和运行期风险预测预警具有借鉴意义。

参考文献 (References)

- 谢立广, 杨群. 关于隧道结构的安全推演及寿命预测 [J]. 公路交通科技, 2021, 38(07): 107-113.

XIE L G, YANG Q. Safety deduction and life prediction of tunnel structure [J]. Journal of Highway and Transportation Research and Development, 2021, 38(07): 107-113. (in Chinese)

- United States Federal Highway Administration Office of Asset Management. Highway and Rail Transit Tunnel Inspection Manual [R]. Washington: U.S. Federal Highway Administration, 2005.

- 罗鑫, 夏才初. 隧道病害分级的现状及问题 [J]. 地下空间与工程学报, 2006, 2(5): 877-880.

LUO X, XIA C C. Current situation and problems of classification of tunnel diseases [J]. Chinese Journal of Underground Space and Engineering, 2006, 2(5): 877-880. (in Chinese)

- 中华人民共和国国家发展和改革委员会. 水工隧洞设计规范: DL/T5195—2004 [S]. 北京: 中国电力出版社, 2004.

National Development and Reform Commission. Specification for Design of Hydraulic Tunnel: DL/T 5195—2004 [S]. Beijing: China Electric Power Press, 2004. (in Chinese)

- 中华人民共和国水利部. 水利水电工程合理使用年限及耐久性设计规范: SL654—2014 [S]. 北京: 中国水利水电出版社, 2014.

Ministry of Water Resources of the People's Republic of China. Code for Rational Service Life and Durability Design of Water Resources and Hydropower Projects: SL654—2014 [S]. Beijing: China Water & Power Press, 2014. (in Chinese)

- 中华人民共和国交通运输部. 公路隧道养护技术规范: JTG H12—2015 [S]. 北京: 人民交通出版社, 2015.

Ministry of Transport of the People's Republic of China. Technical Specification of Maintenance for Highway Tunnel: JTG H12—2015 [S]. Beijing: China Communications Press, 2015. (in Chinese)

- 中华人民共和国水利部. 水工隧洞安全鉴定规程: SL/T 790—2020 [S]. 北京: 中国水利水电出版社, 2020.

Ministry of Water Resources of the People's Republic of China. Code of Hydraulic Tunnel Safety Evaluation: SL/T 790—2020 [S]. Beijing: China Water & Power Press, 2020. (in Chinese)

- 靳春玲. 引大入秦工程隧洞安全评价研究 [J]. 城市道桥与防洪, 2017, (01): 152-154+19.

JIN C L. Study on safety evaluation of tunnel in diversion project from Datong river to Qinwangchuan basin [J]. Urban Roads Bridges & Flood Control, 2017, (01): 152-154+19. (in Chinese)

- 汤雷, 谢炀, 宋人心, 等. 水工隧洞衬砌混凝土质量检测与安全分级评价 [J]. 水利水电技术, 2018, 49(04): 203-209.

TANG L, XIE Y, SONG R X, et al. Quality detection and safety classification evaluation of lining concrete for hydraulic tunnel [J]. Water Resources and Hydropower Engineering, 2018, 49(4): 203-209. (in Chinese)

- 贺思乐, 靳春玲, 祁英弟, 等. 基于RS-可拓云模型的水工隧洞围岩安全评价 [J]. 水电能源科学, 2020, 38(07): 138-141.

HE S L, JIN C L, QI Y D, et al. Safety evaluation of hydraulic tunnel surrounding rock based on RS-Extension Cloud Model [J]. Water Resources and Power, 2020, 38(7): 138-141. (in Chinese)

- 辛萌, 王润, 贾涛, 等. 长期冲蚀作用下水工隧洞衬砌结构的受力分析与安全评价 [J]. 水利与建筑工程学报, 2023, 21(02): 68-73.

XIN M, WANG R, JIA T, et al. Stress analysis and safety evaluation of hydraulic tunnel lining structure under long-term erosion effect [J]. Journal of Water Resources and Architectural Engineering, 2023, 21(02): 68-73. (in Chinese)

- 陆丽丽, 靳春玲, 贡力, 等. 基于改进G2-AEW-UMT模型的引水隧洞结构安全评价 [J]. 水资源与水工程学报, 2020, 31(01): 172-178.

LU L L, JIN C L, GONG L, et al. Structure safety evaluation of diversion tunnel based on improved G2-anti-entropy weight and unascertained measure theory [J]. Journal of Water Resources and Water Engineering, 2020, 31(01): 172-178. (in Chinese)

- 刘万林, 尹 航, 苏 旸, 等. 基于改进的G2-AEW-UMT模型的水工隧洞施工安全风险评价及障碍因子诊断 [J]. 水资源与水工程学报, 2023, 34(03): 184-192.

LIU W L, YIN H, SU Y, et al. Safety risk evaluation and diagnosis of hydraulic tunnel construction based on G2-AEW-UMT Model [J]. Journal of Water Resources and Water Engineering, 2023, 34(3): 184-192. (in Chinese)

- 张翌娜, 陈 磊, 侯 鸽, 等. 基于层次分析-物元可拓-变权理论模型的TBM引水隧洞结构健康评价 [J]. 现代隧道技术, 2024, 61(6): 240-250.

ZHANG Y N, CHEN L, HOU G, et al. Structural health assessment of TBM water diversion tunnels based on the AHP-matter-element extension-variable weight theoretical model [J]. Modern Tunnelling Technology, 2024, 61(6): 240-250. (in Chinese)

- 祁英弟, 靳春玲, 贡力. 基于ANP-灰色关联TOPSIS法的引水隧洞病害安全性评价 [J]. 水资源与水工程学报, 2019, 30(01): 143-149.

QI Y D, JIN C L, GONG L. Safety evaluation of diversion tunnel disease based on ANP-Grey correlation TOPSIS Method [J]. Journal of Water Resources and Water Engineering, 2019, 30(01): 143-149. (in Chinese)

- Zhang S.R., Liu T., Wang C. Multi-source Data Fusion Method for Structural Safety Assessment of Water Diversion Structures[J]. Journal of Hydroinformatics, 2021, 23(2): 249-266.

- 刘康, 刘昭伟, 陈永灿, 等.引水隧洞结构安全风险评价的动态Bayes网络模型 [J]. 清华大学学报(自然科学版), 2023, 63(07): 1041-1049.

LIU K, LIU Z W, CHEN Y C, et al. Dynamic Bayesian network model for the safety risk evaluation of adiversion tunnel structure [J]. Journal of Tsinghua University(Science and Technology), 2023, 63(07): 1041-1049. (in Chinese)

- Liu H.L. Safety state evaluation method of the highway tunnel structure [J]. Heliyon, 2023, 9(6): e17537.

- Zhao L.L., Yang W.B., Wang Z.L., et al. Long-term safety evaluation of soft rock tun-nel structure based on knowledge decision-making and data-driven models [J]. Computers and Geotechnics, 2024, 169: 106244.

- Liu D.J., Zhang W.P., Dai Q.Q., et al. Safety evaluation method for operational shield tunnels based on semi-supervised learning and a stacking algorithm [J]. Tunnelling and Underground Space Technology, 2024, 153: 106027.

- Chen H.Y., Lei Y., Xia L.Y., et al. Dynamic evaluation of the safety risk during shield construction near existing tunnels via a pair-copula Bayesian network [J]. Applied Soft Computing 2025, 169: 112583.

- Jin X., Qiao K.K., Bu M.H., et al. Intelligent safety evaluation of tunnel lining cracks based on machine learning [J]. Engineering Failure Analysis, 2025, 167(B): 109082.

- Liu X.R., Sun W.Z., Zhou X.H., et al. Safety evaluation of in-service sulfate corrosion tunnel considering time effect and free surface evaporation effect [J]. Tunnelling and Under-ground Space Technology, 2025, 155(1): 106165.

- ZHANG L, LIU Y R, YANG Q. A creep model with damage based on internal variable theory and its fundamental properties [J]. Mechanics of Materials, 2014, 78: 44-55.

- 张泷, 刘耀儒, 杨强. 基于内变量热力学的岩石蠕变与应力松弛研究 [J]. 岩石力学与工程学报, 2015, 34(4): 755-762.

ZHANG L, LIU Y R, YANG Q. Creep and relaxation of rock mass based on thermodynamics with internal state varibales [J]. Chinese Journal of Rock Mechanics and Engineering, 2015, 34(4): 755-762. (in Chinese)

- 张泷. 基于内变量热力学的流变模型及岩体结构长期稳定性研究 [D]. 北京: 清华大学, 2015.

ZHANG L. Research on rheological model based on thermodynamics with internal state variables and long-term stability of rock mass structures [D]. Beijing: Tsinghua University, 2015. (in Chinese)

- 毋雪雁, 王水花, 张煜东. K最近邻算法理论与应用综述 [J]. 计算机工程与应用, 2017, 53(21): 1-7.

WU X Y, WANG S H, ZHANG Y D. Survey on theory and application of k-Nearest-Neighbors algorithm [J]. Computer Engineering and Applications, 2017, 53(21): 1-7. (in Chinese)

- 王翔, 陈小鸿, 杨祥妹. 基于K最近邻算法的高速公路短时行程时间预测 [J]. 中国公路学报, 2015, 28(01): 102-111.

WANG X, CHEN X H, YANG X M. Short term prediction of expressway travel time based on K nearest neighbor algorithm [J]. China Journal of Highway and Transport, 2015, 28(01): 102-111. (in Chinese)

- 殷亚博, 杨文忠, 杨慧婷, 等. 基于卷积神经网络和KNN的短文本分类算法研究 [J]. 计算机工程, 2018, 44(07): 193-198.

YIN Y B, YANG W Z, YANG H T, et al. Research on short text classification algorithm based on convolutional neural network and KNN [J]. Computer Engineering, 2018, 44(07): 193-198. (in Chinese)

- PANG Z Y, HUANG Y Q, XIE M W, et al. A physics-informed framework for mechanical parameters inversion in tunnel engineering based on deep learning and Bayesian optimization [J]. Tunnelling and Underground Space Technology, 2025, 166: 106940.

Safety assessment of tunnel structure based on real-time inversion and deformation evolution

Abstract: [Objective] With the continuous expansion and increasing complexity of water diversion tunnels in hydropower projects, ensuring their long-term structural safety has become a critical engineering challenge. Conventional safety evaluation methods often rely on qualitative assessments or multi-index systems, which are highly subjective and fail to adequately account for progressive material deterioration and time-dependent deformation. This study aims to propose an integrated quantitative framework based on real-time inversion of mechanical parameters and deformation evolution analysis to dynamically evaluate the structural safety of tunnels. [Methods] The proposed framework integrates four major components: data preprocessing, parameter inversion, deformation simulation, and safety evaluation. First, raw deformation monitoring data are preprocessed using the K-Nearest Neighbors (KNN) algorithm for imputing missing values, a sliding-window Z-score method for outlier identification and correction, and logarithmic trend fitting for noise reduction. Second, a physics-informed inversion approach combining deep learning architectures—Fully Connected Layer(FCL) and Gated Recurrent Unit(GRU)—with Bayesian optimization is established to infer the current mechanical parameters of the tunnel based on preprocessed deformation data. Third, an elasto-viscoplastic damage creep constitutive model founded on internal variable thermodynamics is employed to simulate the deformation behavior under various material degradation scenarios, represented by different strength reduction coefficients (Kᵣ). Finally, based on the analysis of material creep behavior and deformation evolution patterns, a time-dependent "3S" safety evaluation index system is established, comprising the long-term deformation acceleration safety coefficient (S₁), the nonlinear deformation initiation safety coefficient (S₂), and the short-term deformation acceleration safety coefficient (S₃). The safety state of the tunnel structure is quantified according to deformation evolution characteristics using the proposed 3S safety coefficients. The physical implications of these indices are elaborated as follows: S₁ characterizes the critical point at which the structure transitions into an accelerated creep phase under continuous strength attenuation, indicating long-term instability risk; S₂ reflects the onset of deviation from linear response during initial deformation, marking the beginning of dominance by nonlinear mechanical behavior; and S₃ evaluates the threshold condition for significant acceleration of deformation within a defined short-term period, serving as an indicator of potential sudden instability. [Results] The proposed method was implemented in the JLL Tunnel, a 20 km-long underground structure located in Hunan Province, China, which features complex geological conditions. Mechanical parameters were successfully inverted from field monitoring data, with simulated deformation curves showing high agreement with measured values. Numerical simulations under different Kᵣ conditions revealed distinct deformation patterns: when Kᵣ≥0.7, deformation stabilized after initial convergence; within the range of 0.4≤Kᵣ≤0.6, deformation exhibited slow growth followed by an acceleration phase; and when Kᵣ≤0.3, significant acceleration occurred within a short time period. The computed 3S safety coefficients were S₁=2.4–3.0, S₂=3.7–4.6, and S₃=5.7–7.3, indicating that the tunnel is currently in a safe state with sufficient safety margins. These results validate the method’s effectiveness in distinguishing between short-term and long-term risks and enabling safety early-warning based on deformation trajectory analysis. [Conclusions] This study proposes an integrated quantitative framework for tunnel structural safety evaluation that effectively combines real-time monitoring data, physics-based modeling, and deformation evolution analysis. The established 3S index system provides refined insight into structural behavior under material degradation and enables safety assessment across multiple time scales. Compared with conventional methods, the proposed framework enhances objectivity, supports dynamic prediction of time-dependent performance, and facilitates lifecycle safety management and preventive maintenance of tunnel structures. The methodology demonstrates strong generalizability and offers significant practical value for risk prevention and sustainable operation in tunnel engineering.

Key words: tunnel; safety assessment; mechanical parameters inversion; deformation evolution; material degradation